Current job status:

- All bugs of PCB and schematic were eliminated.

- The PCB works correct

- The program done for a 97%.

Next components were done.

- For the host PC:

| The sound bank compiler | Compile number os source files to one sound bank file (1). |
| --- | --- |
| The file server | Make your PC as external hard drive for sndsynt device (2). |
| Sound control tool | App for PC withc lets control sndsynt with CakeWalk |

- What is done:

| Lib Sdramdrv | Driver SDRAM |
| --- | --- |
| Lib Synt | Sound render |
| Lib Banks | Access to sound and midi banks (for compiled file format (1)) |
| Lib Seqp | MIDI events |
| Lib Seq | MIDI files |
| Lib Midi | Hardware MIDI interface |
| Lib Terminal | Keyboard and display |
| Lib DataFlash | Access to FLASH memory |
| Lib Fcodecdrv | Control of DAC PCM1717 |
| Lib FileIO | Access to the host files (to use server (2)) |
| Lib DataFlashFileIO | The file system in a FLASH |
| MainProgram | User’s program |

In a progress:

| lib Games | Generator of game’s sequences |
| --- | --- |
| lib Script | Shell |
| lib Control | The game control interface |

Current version of program simulate common MIDI synthesizer. It can play internal MIDI file as well. The sndsynt downloads sound bank from PC or from DATAFLASH.
Number of channels 16 - 32 it depend from complexity sounds and FX. By default there is 21 channel.